DOT Series LED Blank-out Sign in IP66/NEMA 4X Cabinet

Outdoor/indoor blank-out signs with an IP66/NEMA 4X cabinet, narrow angle LEDs and contrasting black face. The sign message is formed by visible LEDs that illuminate when activated and blank-out when deactivated.

| Electrical | Standard configuration | Notes |
| --- | --- | --- |
| Input voltage | 100-240VAC | Optional: 9-24VDC |
| Control type | Switch or external relay |  |
| Number of inputs | One input (Line or +) per message One common (Neutral or -) One Earth Ground |  |
| Input connection type | Stripped end, 18AWG wire | Options available (below) |
| Load type | Capacitive | Components in a system designed to control resistive loads (e.g., an incandescent lamp) may not be suitable. |
| Power supply efficiency (@120VAC, 50% rated load) | 81% | Power supply shall not be loaded beyond its rated safe operating area |
| Surge protection | None | Optional: DIN Rail mounted surge protectors |
| Power supply configuration | Non-redundant | Optional: Redundant power supplies |
| UL/cUL Listed | Yes (for wet locations) |  |
| Physical/Environmental | Standard configuration | Notes |
| IP/NEMA rating | IP66/NEMA 4X | All in-field modifications/conduit entry points must be properly sealed |
| Temperature and Humidity (storage and operation) | -40°F to +165°F 0% to 99% (non-condensing) |  |
| Face material | 0.125” thick, tinted, matte, impact-modified acrylic |  |
| Cabinet construction | 0.125” thick 6063-T6 aluminum alloy extruded cabinet with welded seams, stainless steel hardware and hinged face(s). All exterior seams shall be continuous MIG welded to form a single structure. The hinged face(s) shall be attached to cabinet using a full-length extruded hinge. The cabinet face is flanged on all sides to shed water. The door closes around its flanged frame and is compressed against a closed-cell silicone gasket. |  |
| Cabinet depth | Single-faced: 6.50” |  |
| Cabinet finish | Powder-coated black |  |
| Cabinet interior | Conformal coated circuit board, NEMA 4X breather drain (for atmospheric pressure equalization), sealed/closed-end fasteners used for all bolt-on mounting. |  |
| Mounting | Two horizontal rows of M8x1.25 sealed Rivnuts; Rows are centered and spaced apart ⅔x the sign’s height (spacing, quantity, and number of rows may vary) | Options available (below) |

| Visual/Illumination | Standard configuration | Notes |  |
| --- | --- | --- | --- |
| LED drive current | 15mA per string | String length varies per color |  |
| Automatic Photodimming | Single photocell, IR-sensitive, gradual brightness adjustment | Optional: Redundant photocells |  |
| Message flashing | Adjustable speed/pattern, set from factory | OFF by default |  |
| Message face | Black message board protected by tinted, matte (glare-reducing) window | See Physical/Environmental for face construction |  |
| LED viewing angle | Narrow 30° cone (15° from center axis) | Optional: Wide-angle 120° cone |  |
| LED brightness (mcd, typ.) |  |  |  |
| Color (typ. wavelength/temperature) | Narrow-angle | Wide-angle |  |
| Red (625nm) | 3750 | 660 |  |
| Amber (589nm) | 3900 | 780 |  |
| Green (525nm) | 7500 | 1200 |  |
| Blue (470nm) | 1500 | 210 |  |
| White (5500K-6500K) | 4350 | 1375 |  |
| Options |  |  |  |
| Input voltage | 9-24VDC |  |  |
| Input connection type | Stripped end, 18AWG wire |  |  |
|  | Tool-actuated terminal blocks (DIN rail) |  |  |
|  | Lever-actuated terminals (DIN rail) |  |  |
| Surge protection | None |  |  |
|  | 20kA DIN rail mounted surge protectors |  |  |
| Power supply configuration | Single |  |  |
|  | Redundant (at least two per message) |  |  |
| Hood/Visor | None |  |  |
|  | 6” depth (from face) |  |  |
| Mounting | Rivnut array |  |  |
|  | C-Channel (bolt-on) |  |  |
|  | Mounting tabs |  |  |
|  | SkyBracket mast arm mount (vertical or horizontal) |  |  |
|  | Skybracket top/bottom elbow mount (with locking, serrated ring) |  |  |
|  | Integrated locking, serrated ring (top and bottom) |  |  |
| Photocells | Single |  |  |
|  | Redundant (x3) |  |  |
| Message flashing | None (adjustable on photodimmer board) |  |  |
|  | Custom (pre-set from factory) |  |  |
| LED viewing angle | Narrow 30° cone (15° from center axis) |  |  |
|  | Wide 120° cone (60° from center axis) |  |  |