SBL Series Indoor Blank-out LED Backlit Sign

The SBL Series Indoor, Blank-out, LED Backlit sign is designed with a matrix of energy efficient LEDs illuminating a block-style message. The sign features blank-out technology that only displays the sign message when illuminated.

It is engineered with a solid-state circuit board and switch-mode power supply for maximum LED life. The cabinet is constructed of corrosion-resistant, extruded aluminum and ships with our built-in Lift'N Shift back for wall mounting. Ceiling, post, wall-projection and recessed mountings are available. Standard cabinet finish is Duranodic Bronze; custom paint finishes are available. Sign can be controlled by a switch, Signal-Tech Sign Control Software or third-party relay. Control switch and software can be purchased separately. UL/cUL Listed for wet locations.

Technical Specifications

| Illumination | Backlit with LEDs |
| --- | --- |
| LED Message Colors | Red, Amber, Green, Blue and White |
| Message | Standard Font: Swiss 721 Bold BT Custom font styles, monochrome symbols and graphics available Message blanks-out when off. |
| Electrical | Input voltage 120-277VAC or low volt 12-24VDC UL/cUL Listed for wet locations |
| Cabinet Construction Single-faced signs; double-faced signs up to 14”H x 18”W Double-faced signs larger than 14”H x 18”W | One-piece extruded aluminum frame with an 1/8” thick, dark grey diffused acrylic face panel Available but requires custom engineering |
| Cabinet Depth | Single-faced: 2 ½” Double-faced up to 14”H x 18”W: 5 ½” Double-faced over 14”H x 18”W: 7 ½” |
| Cabinet Finish | Standard: Duranodic Bronze Options: Custom paint colors or color match paint |
| IP/NEMA Rating | IP55/NEMA 3RX |
| Control | Switch, Signal-Tech Sign Control Software or external relay |
| Accessories/Options | Flashing, Audible Alarm |
| Warranty | 2 Years |
| Country of Origin | USA |
| Mounting | Lift’N Shift sign back offers built in wall mounting (single faced signs only) Additional mounting can purchased separately: Ceiling, post, recessed |