# Signal-Tech SNS Series Specifications

Backlit, retroreflective street name signs

## Main Features

- Superior visibility, day and night:

  - Translucent, prismatic (retroreflective) printed face

  - Edge-lit with cool white LED modules for uniform lighting and contrast

  - Minimum brightness: 105 nits (cd/m2)

  - Optional photocell for automatic backlight control

- Safe, quick, and effortless installation:

  - External wiring connection point and tri-stud mounting hat

  - Lockable latches

  - Bottom-hinging face with holding cable

  - Lightweight (4.5-6 lbs. per ft²)

- Built to last:

  - Reflective graphics applied on second surface of UV-resistant face

  - 0.090” cabinet wall thickness with durable black finish

  - Uses high-quality, IP-66+ rated electrical components

## Specifications

| Dimensions and Weight |  |  |
| --- | --- | --- |
| Height | 18” or 24” | Visual opening: 16” or 22” |
| Width | 72”, 96”, or 120” (6’, 8’, or 10’) | Visual opening: 70”, 94”, or 118" |
| Depth | 5.25” | Single- or double-faced |
| Weight per ft2 | 4.5-6 lbs. | Single- or double-faced; see product spec sheet for exact weight |
| Visual |  |  |
| Minimum brightness | 105 nits (cd/m2) | Units less than 24” tall will appear brighter |
| Print/graphics | Translucent, Retroreflective Sheeting |  |
| Standard face color | Printed PMS 342 (Green) | Other standard colors: Blue: PMS 294 Brown: PMS 469 |
| Font | Highway Gothic | Series C or D used, depending on size and text; Other fonts available upon request |
| Cabinet finish | Low-gloss black | Default—others available upon request |
| Electrical |  |  |
| Input voltage | 100-277VAC |  |
| Power (nominal) | 30W, 40W, or 50W | 72”, 96”, or 120” length |
| Control | Optional photocell | Sign will illuminate automatically at night |
| External cable | 2’ length; 14AWG, 3-conductor w/ gray PVC jacket | Junction box not included; internal strain relief collar |
| Physical Construction |  |  |
| Component IP Ratings | IP66 (Photocell assembly) IP67 (Power supply) IP68 (LED modules) | Cabinet is designed to meet IP55/NEMA 3RX standards |
| Cabinet material | 6063-T6 Aluminum alloy (0.090” wall) |  |
| Face material | UV-resistant, 0.125” Impact-modified acrylic |  |
| Mounting interface | 3/16” thick Aluminum with tri-stud mounting pattern | Mounting interface comes standard |